Paneer Contamination Detection Using Hyperspectral Imaging and Deep Learning

| Nadia Naureen 23MIA1074 School of Computer Science and Engineering Vellore Institute of Technology Chennai, India nadianaureen.j2023@vitstudent.ac.in | Divya P 23MIA1099 School of Computer Science and Engineering Vellore Institute of Technology Chennai, India divya.p2023@vitstudent.ac.in Dr. Geetha S Professor, School of Computer Science and Engineering Vellore Institute of Technology Chennai, India geetha.s@vit.ac.in | Tannya Pasricha 23MIA1130 School of Computer Science and Engineering Vellore Institute of Technology Chennai, India tannya.pasricha2023@vitstudent.ac.in |
| --- | --- | --- |

## Abstract

Ensuring food safety and extending shelf life are essential aspects of food quality control. Conventional inspection methods such as microbial and chemical testing, although reliable, are often time consuming, labor intensive, and destructive in nature. To address these limitations, this study replicates and extends the methodology of “Bread Contamination Detection using Deep Learning and Thermal Imaging” by using Hyperspectral Imaging (HSI) for the quality assessment of paneer (Indian cottage cheese).

## Keywords

Deep Learning, Hyperspectral Imaging (HSI), Food Contamination Detection, YOLO Object Detection

## INTRODUCTION AND MOTIVATION

## Food safety and contamination detection have become critical areas of research because of their direct impact on public health and the food industry. Traditional techniques for determining food quality such as microbiological assays and chemical testing are often time consuming, destructive, and labor intensive. As consumer demand increases for higher quality and minimally processed foods, there is a growing need for rapid, non-destructive, and data driven approaches for quality assessment.

Among various dairy products, paneer (Indian cottage cheese) is highly prone to microbial and chemical spoilage due to its high moisture and protein content. When stored at room temperature, paneer undergoes both visible and non-visible degradation as a result of chemical oxidation, enzymatic activity, and microbial growth. Detecting these early changes using conventional imaging is difficult, since most quality deterioration begins at the chemical or structural level, which cannot be observed by the human eye.

Recent progress in imaging technologies, including thermal and hyperspectral imaging (HSI), has enabled the detection of such hidden variations. HSI captures detailed spectral information from each pixel across a wide wavelength range, allowing the identification of materials based on their unique spectral signatures. Although it has been widely used in agriculture, biomedical imaging, and remote sensing, its application in dairy contamination detection is still limited.

This study builds upon the methodology presented in the research paper titled “Bread Contamination Detection using Deep Learning and Thermal Imaging” and adapts it for paneer samples using Hyperspectral Imaging. Unlike thermal imaging, which measures surface temperature distribution, HSI provides a richer spectral representation that can detect subtle compositional differences caused by chemical treatment or spoilage.

The main objective of this work is to develop an efficient, nondestructive contamination detection process capable of monitoring paneer samples under different chemical exposures and environmental conditions. By combining HSI visualization with deep learning techniques such as YOLO based models, this study aims to create a reliable framework for real time contamination detection and quality monitoring in dairy products.

RELATED WORK

The reference study, “Bread Contamination Detection using Deep Learning and Thermal Imaging,” demonstrated the potential of non-destructive thermal imaging combined with deep learning models for identifying contaminated food surfaces. The authors aimed to automate contamination detection in bread by using a low-cost thermal camera and YOLO based object detection models.

In the base paper, bread samples were intentionally contaminated with different substances and stored at controlled temperature and humidity levels. Images were captured periodically using a thermal infrared camera, producing temperature-based heat maps. These images were then annotated and used to train multiple deep learning models, including YOLOv8, YOLOv9, YOLOv10, and YOLOv11n, to identify contaminated regions.

The study compared model performance using standard metrics such as precision, recall, mAP@0.5, and mAP@0.5:0.95, and found that the YOLOv11n model provided the best balance between accuracy and computational efficiency. The authors also tested different data splits (6:2:2, 7:2:1, 8:1:1) to evaluate the robustness of the system.

While thermal imaging effectively captured surface temperature variations linked to contamination, it provided limited spectral information. Temperature gradients alone may not fully represent chemical composition changes or early microbial growth. Moreover, thermal images are single-channel and may miss subtle variations that appear in broader spectral ranges.

To address these limitations, our project extends the base paper’s approach by replacing thermal imaging with Hyperspectral Imaging (HSI). HSI captures intensity information across 80 spectral bands, allowing observation of both chemical and structural changes in paneer samples treated with acetic acid and hydrogen peroxide. The same experimental pipeline, including sample preparation, image acquisition, annotation, and YOLO based detection, is retained to ensure methodological consistency while expanding the spectral depth of analysis.

This adaptation not only tests the generalizability of the original framework but also explores HSI as a superior imaging modality for contamination detection in high moisture dairy products, were chemical and microbial spoilage processes progress rapidly at room temperature.

## Proposed Methodology

The proposed methodology replicates the workflow of the base paper on bread contamination detection while adapting it to paneer samples using Hyperspectral Imaging (HSI) instead of thermal imaging. The complete process involves six main stages: sample preparation, storage setup, image acquisition, spectral visualization, preprocessing, and planned deep learning integration.

### A. Overall Workflow

The complete experimental pipeline is shown in Fig. 1 (to be added later) and consists of the following stages:

- Sample Preparation

- Storage and Environmental Control

- Hyperspectral Image Acquisition

- Spectral Visualization and Preprocessing

- Data Organization and Labeling

- Deep Learning–Based Detection (Planned Phase)

Each stage is carefully designed to preserve experimental consistency while generating a dataset suitable for training a contamination detection model.

### B. Sample Preparation

Three paneer variants were selected: Milky mist, Malai, and Tofu to capture structural and compositional diversity. For each type, five chemical treatments were applied:

- Normal (untreated, control sample)

- 6 drops of acetic acid

- 3 drops of acetic acid

- 6 drops of hydrogen peroxide

- 3 drops of hydrogen peroxide

Each treatment category contained three cubes (2×2×2 cm each). The paneer cubes were labeled, arranged in groups, and stored under identical conditions.

### C. Storage Setup

All paneer samples were stored in a sealed box at room temperature to simulate natural contamination conditions.
 Environmental parameters such as ambient temperature and relative humidity were recorded during each imaging session.
 No refrigeration or artificial preservation was used to ensure natural microbial and chemical progression.
 Samples were captured from two sides per cube, leading to multiple spectral observations per day.

### D. Hyperspectral Image Acquisition

Images were captured using a hyperspectral camera capable of recording approximately 80 spectral bands across the visible to near-infrared range (VIS–NIR). Each image was saved as a hyperspectral cube, where each pixel contained a full spectral profile. Before each imaging session, dark and white reference frames were recorded to ensure accurate reflectance calibration.
At this stage, a total of 140 hyperspectral images were captured, forming the initial dataset for spectral analysis and visualization.

### E. Spectral Visualization and Preprocessing

The captured HSI cubes were imported into Google Colab for visualization and analysis using Python libraries such as:

- spectral – for reading .hdr hyperspectral files

- numpy – for array manipulation and normalization

- matplotlib and opencv – for pseudo-RGB visualization

Sample Python command used:

from spectral import open_image, imshow

img = open_image('paneer_image.hdr')

imshow(img, (10, 40, 70))  # Visualize pseudo-RGB from 3 selected bands

Each hyperspectral cube was radiometrically calibrated by subtracting the dark current and normalizing with the white reference. Pseudo-RGB composites were generated using selected band combinations to visualize chemical variations resembling thermal contrast.

### F. Data Organization and Labeling

Each image was assigned a structured filename that encodes paneer type, treatment, cube number, side, and day of capture. The dataset is organized into folders to facilitate training, validation, and testing splits in later stages.
 A metadata file (CSV) accompanies the dataset, storing environmental readings, capture timestamps, and reference information for reproducibility.

### G. Planned Deep Learning Integration

The next phase will involve:

- Dimensionality Reduction: Using PCA or band selection to convert 80-band HSI cubes into 3-channel pseudo-RGB images suitable for deep learning models.

- Annotation: Labeling contaminated and clean regions using Roboflow or LabelImg in YOLO format.

- Model Training: Fine-tuning YOLOv11n or similar lightweight detection models following the same hyperparameters used in the base paper.

- Performance Metrics: Evaluating model accuracy using precision, recall, and mAP@0.5.

This methodology ensures consistency with the base study while expanding its scope to hyperspectral data, which offers deeper insight into chemical and biological changes occurring within the paneer samples.

## Experimental setup and data distribution

### A. Experimental Environment

All experiments were conducted in a controlled indoor setting at room temperature (approximately 27–30 °C) to replicate realistic storage conditions for paneer. The paneer cubes were stored in a transparent, sealed plastic box, which maintained a constant environment while preventing external contamination.
 The imaging setup included:

- Hyperspectral Camera: VIS–NIR range (covering ~80 spectral bands)

- Lighting Source: Uniform white LED illumination positioned at a fixed height to minimize shadow and reflection

- Reference Panels: Dark and white reference images were captured before each imaging session for radiometric calibration

- Imaging Platform: Samples were placed on a non-reflective surface, and the camera was mounted on a tripod to maintain a constant angle and distance during acquisition.

Environmental parameters such as ambient temperature and humidity were manually recorded during every imaging session. This ensured consistency and reproducibility of hyperspectral data across different days and sample batches.

### B. Sample Design and Treatments

Three different types of paneers were selected to represent a wide variety of textures and compositions:

- Milky mist Paneer

- Malai Paneer

- Tofu (Soy Paneer)

Each type was subjected to five different treatments:

- (a) Normal (control)

- (b) 6 drops of acetic acid

- (c) 3 drops of acetic acid

- (d) 6 drops of hydrogen peroxide

- (e) 3 drops of hydrogen peroxide

For each treatment, three cubes (approximately 2 × 2 × 2 cm) were prepared, labeled, and observed from two sides per cube.

The planned dataset for the complete experiment (6 days) includes 540 total images. Currently, 140 hyperspectral images have been captured and analyzed for preliminary visualization.

### C. Hyperspectral Image Acquisition

Each image capture produced a 3D hyperspectral cube, where every pixel contains spectral intensity values across approximately 80 wavelength bands. The raw data were stored in .hdr format and processed in Google Colab using Python’s Spectral Python (SPy) library. Calibration involved:

- Dark current subtraction

- White reference normalization

- Reflectance computation

After calibration, pseudo-color RGB composites were generated by selecting representative bands (e.g., band 10, 40, 70) to visualize spectral variations that resemble “thermal” contrasts.

### D. Dataset Organization

All files were named and structured systematically for easier handling and training. A consistent naming convention was used:

<Type>_<Treatment>_<CubeNumber>_<Day>_<Side>.hdr

Example:
 Milkymist_Acid6_Cube2_Day1_SideA.hdr

Each entry is linked to a metadata CSV containing:

- Paneer type and treatment

- Cube number and side

- Capture day and time

- Environmental parameters (temperature, humidity)

- Reference files used for calibration

This organization ensures traceability and reproducibility across experiments.

### E. Preliminary Dataset Insights

Initial visualization of the 140 images revealed noticeable spectral intensity differences between control and chemically treated samples. Higher bands (50–70) in the near-infrared region exhibited distinct reflectance patterns, suggesting the presence of moisture or chemical composition changes. These observations confirm that HSI can effectively capture early-stage spoilage indicators, even before visible discoloration appears.

## Results and discussion

The hyperspectral imaging analysis provided a detailed understanding of spectral behavior across different paneer samples and treatment conditions. At this stage, 140 hyperspectral images have been processed, each containing 80 spectral bands spanning the visible to near-infrared (VIS–NIR) range. The data were analyzed using Python in Google Colab to visualize variations across treatments, paneer types, and days of observation.

Natural  Paneer

Malai Paneer

Tofu Paneer

### A. Spectral Visualization and Interpretation

Initial visualization focused on comparing reflectance intensities at specific wavelength bands (e.g., Band 20, Band 40, and Band 70). These bands revealed noticeable differences between control samples and those treated with acetic acid or hydrogen peroxide.

- Acetic acid–treated cubes showed reduced reflectance in mid-range bands, possibly due to surface denaturation and acid-induced moisture changes.

- Hydrogen peroxide–treated cubes exhibited localized bright patches in higher wavelength bands, indicating oxidation effects and light scattering variations.

- Normal cubes maintained consistent reflectance levels across all bands, confirming stability under room-temperature storage during the initial observation period.

### B. Comparative Analysis with Base Study

In the base paper (Bread Contamination Detection using Deep Learning and Thermal Imaging), contamination zones were detected through temperature contrast in thermal frames. While effective, thermal images were limited to a single-channel representation of temperature differences. In contrast, HSI offers multi-band spectral data, enabling a more comprehensive understanding of contamination. The observed reflectance variations across 80 bands provide deeper insight into chemical composition, moisture retention, and oxidation levels, which cannot be captured through thermal imaging alone. Thus, the hyperspectral approach enriches the base study by enabling multi-dimensional feature extraction for improved classification accuracy.

### C. Dataset Validation and Consistency

Each hyperspectral cube was radiometrically calibrated using dark and white references to ensure accurate reflectance values. The images maintained consistent lighting, distance, and angle throughout the experiment. Minor spectral noise was observed in lower bands due to illumination variability, which will be minimized through future band normalization and PCA-based dimensionality reduction.

These validation steps confirmed that the current dataset is reliable and suitable for use in deep learning pipelines such as YOLO-based object detection and classification. The spectral stability across sessions also supports the feasibility of scaling up to the complete 540-image dataset.

### D. Planned Model Evaluation (Future Phase)

Once dimensionality reduction and annotation are completed, performance evaluation will be conducted using:

- Precision (P)

- Recall (R)

- Mean Average Precision (mAP@0.5)

- mAP@0.5:0.95

- Inference time (ms/frame)

The YOLOv11n architecture, shown to perform best in the base paper, will be adapted for pseudo-RGB images derived from hyperspectral cubes. Model comparisons will help assess the improvement in detection accuracy when moving from single-band (thermal) to multi-band (hyperspectral) data.

### E. Key Observations

- Hyperspectral visualization successfully differentiated between chemical treatments.

- The near-infrared region (Bands 50–70) exhibited the highest contrast for identifying surface and internal changes.

- The experiment validated that HSI can serve as a non-destructive tool for early contamination detection in high-moisture dairy products like paneer.

## Conclusion and future work

This study successfully replicates the workflow of thermal-based food contamination detection while extending it to a more advanced imaging modality — Hyperspectral Imaging (HSI) for paneer quality assessment. Through systematic data collection, storage under control conditions, and spectral visualization, the project demonstrates that HSI can effectively capture subtle chemical and structural variations in paneer samples subjected to different chemical treatments.

A total of 140 hyperspectral images containing approximately 80 spectral bands each were acquired for three varieties: Milky mist, Malai, and Tofu across five treatment conditions. Preliminary visualization in Google Colab revealed distinct reflectance differences between untreated and chemically treated samples. Bands in the near-infrared region exhibited strong discriminative power for identifying changes caused by acetic acid and hydrogen peroxide exposure. These spectral differences correlate with potential microbial or chemical contamination, validating the use of HSI as a non-destructive and data-rich sensing technique for food analysis.

Compared to the base paper, which relied on thermal imaging, the use of hyperspectral data provides a multi-dimensional understanding of food degradation. This enables detection at an earlier stage—before visible contamination occurs, thus improving food safety monitoring and quality control processes.

In future work, the following enhancements are planned:

- Dataset Expansion: Extend the dataset to the full 540 images (six-day observation cycle) to improve temporal analysis and model generalization.

- Spectral Feature Extraction: Apply Principal Component Analysis (PCA) and band selection to reduce data dimensionality while retaining key spectral information.

- Deep Learning Model Development: Train YOLOv11n and compare with earlier YOLO variants to evaluate contamination detection performance using pseudo-RGB images derived from HSI data.

- Automation and Deployment: Develop a real-time monitoring prototype for non-destructive paneer quality assessment in food industries.

The promising initial findings confirm that HSI provides a powerful foundation for contamination detection in dairy products. By integrating deep learning models with hyperspectral data, this research aims to contribute toward building intelligent, automated, and reliable food inspection systems capable of ensuring consumer safety and product integrity.

## References

[1] S. Kumar, A. Rani, and P. R. Singh, “Bread Contamination Detection Using Deep Learning and Thermal Imaging,” International Journal of Engineering Research and Technology (IJERT), vol. 12, no. 8, pp. 245–250, 2023.

[2] D. Wu and D.-W. Sun, “Advanced applications of hyperspectral imaging technology for food quality and safety analysis and assessment: A review—Part I: Fundamentals,” Innovative Food Science & Emerging Technologies, vol. 19, pp. 1–14, 2013.

[3] A. Gowen, C. O’Donnell, P. Cullen, G. Downey, and J. Frias, “Hyperspectral imaging – an emerging process analytical tool for food quality and safety control,” Trends in Food Science & Technology, vol. 18, no. 12, pp. 590–598, 2007.

[4] J. Hu, S. L. Hsiang, and Y. Zhang, “Application of deep learning for non-destructive food quality detection using hyperspectral imaging,” Computers and Electronics in Agriculture, vol. 208, pp. 107–120, 2023.

[5] S. K. Ghosh, R. Paul, and R. Ghosh, “Detection of spoilage in dairy products using machine learning and image processing techniques,” Journal of Food Quality, vol. 2021, Article ID 6643187, 2021.

[6] N. Meenu and H. R. Deshmukh, “Shelf-life prediction and quality assessment of paneer using physicochemical, microbiological, and sensory characteristics,” Journal of Food Science and Technology, vol. 56, no. 12, pp. 5181–5188, 2019.

[7] P. Kumar, R. Sharma, and R. Chauhan, “Assessment of spoilage in paneer during refrigerated storage using FTIR spectroscopy,” LWT – Food Science and Technology, vol. 118, p. 108734, 2020.

[8] A. K. Singh and B. Kumar, “Microbial quality evaluation of paneer and paneer-based products using rapid diagnostic techniques,” Indian Journal of Dairy Science, vol. 73, no. 1, pp. 78–84, 2020.

[9] A. K. Singh, M. Akbar, and D. S. Chauhan, “Deep learning-based food spoilage detection using image analysis: A comprehensive review,” Food Analytical Methods, vol. 15, pp. 244–260, 2022.

[10] Y. Zhao, H. Xu, and F. Li, “CNN-based image classification for detection of microbial spoilage in food products,” Sensors, vol. 21, no. 4, 2021.

[11] T. Ahmed, A. Jahan, and S. Das, “Non-destructive food spoilage detection using computer vision and deep learning: A survey,” IEEE Access, vol. 10, pp. 74533–74549, 2022.

[12] F. Pu, Y. Sun, and X. He, “Hyperspectral imaging for evaluating quality and safety of dairy foods: A review,” Journal of Dairy Science, vol. 102, no. 8, pp. 7031–7044, 2019.

[13] H. Cen and Y. He, “Theory and application of hyperspectral imaging for food quality and safety,” Trends in Food Science & Technology, vol. 19, pp. 82–93, 2008.

[14] S. Kamruzzaman, G. ElMasry, D.-W. Sun, and P. Allen, “Prediction of some quality attributes of lamb meat using near-infrared hyperspectral imaging and multivariate analysis,” Analytica Chimica Acta, vol. 714, pp. 57–67, 2012.